Mamdani Fuzzy Inference System – Lab Report

1. Problem Description

A delivery robot decides its speed based on three inputs: visibility (0-100%), obstacle distance (0-10m), and battery level (0-100%). The output is a recommended speed (0-10 km/h). Fuzzy logic is used because sensor readings are imprecise and real-world categories like 'close obstacle' are vague.

2. Input and Output Variables

Visibility: low (trap 0,0,20,45), medium (tri 25,50,75), high (trap 55,80,100,100)

Distance: close (trap 0,0,1.5,3.5), medium (tri 2,5,8), far (trap 6,8,10,10)

Battery: low (trap 0,0,20,40), medium (tri 25,50,75), high (trap 60,80,100,100)

Speed output: very_slow (trap 0,0,1,2.5), slow (tri 1.5,3,4.5), moderate (tri 3.5,5.5,7.5), fast (trap 6.5,8,10,10)

3. Rule Base

R1: IF visibility is low OR obstacle is close THEN very slow

R2: IF visibility is medium AND obstacle is close THEN very slow

R3: IF visibility is medium AND obstacle is medium THEN slow

R4: IF visibility is high AND obstacle is medium THEN moderate

R5: IF visibility is high AND obstacle is far AND battery is high THEN fast

R6: IF battery is low AND obstacle is far THEN moderate

R7: IF battery is low AND obstacle is close THEN very slow

R8: IF visibility is medium AND battery is medium AND obstacle is far THEN moderate

R9 (added): IF visibility is high AND battery is low AND obstacle is far THEN moderate

R1 uses OR because either condition alone is dangerous enough to force minimum speed. R5 requires all three conditions to be good before allowing fast speed. R6 limits speed even on a clear path when battery is low to avoid sudden power loss.

4. Mamdani Inference Mechanism

Fuzzification: each crisp sensor value is mapped to membership degrees using the membership functions above.

Rule activation: each rule's antecedent is evaluated using min for AND and max for OR, giving a strength between 0 and 1.

Implication: the output set for each rule is clipped at the rule's activation strength.

Aggregation: all clipped output sets are combined using max.

Defuzzification: the crisp speed is the centroid of the aggregated set: sum(x * mu(x)) / sum(mu(x)).

5. Experiments

visibility=20, distance=1.0, battery=90 -> 0.92 km/h (close obstacle forces minimum speed)

visibility=90, distance=9.0, battery=90 -> 8.60 km/h (all conditions ideal)

visibility=70, distance=4.0, battery=30 -> 4.95 km/h (good visibility offset by low battery)

visibility=45, distance=7.5, battery=55 -> 5.06 km/h (balanced conditions)

visibility=80, distance=2.0, battery=80 -> 0.99 km/h (close obstacle overrides good visibility)

visibility=30, distance=8.0, battery=20 -> 3.80 km/h (poor visibility and low battery conflict with safe distance)

6. Critical Analysis

The system handles extreme inputs well. A close obstacle always produces near-zero speed regardless of other inputs. Conflicting inputs produce intermediate speeds rather than forcing a binary choice.

Weaknesses: membership function boundaries are chosen manually and have a large effect on results (Task 2 showed a 1 km/h change from widening one trapezoid). There are gaps in rule coverage and no ability to learn from data.

7. Possible Improvements

1. Use data from real sensors to set membership function boundaries instead of guessing.

2. Add more rules to cover missing input combinations.

3. Use ANFIS to tune the membership functions automatically from training data.

4. Add more inputs such as floor surface type or payload weight.

5. Test in a robot simulator with noisy sensors before real deployment.

Discussion Questions (Section 15)

1. Why is a fuzzy rule-based system useful in uncertain environments?

Crisp rules break down near thresholds when sensor data is noisy. A fuzzy system assigns partial membership to multiple categories at once, so small input changes produce smooth output changes instead of abrupt jumps.

2. What is the role of membership functions?

They convert a crisp number into degrees of membership in linguistic categories like 'low' or 'close'. This lets the system reason with vague concepts instead of exact numbers.

3. What does it mean for a rule to be partially activated?

The rule's antecedent evaluates to a value between 0 and 1. The consequent set is clipped at that value, meaning the rule contributes partially to the final output rather than fully or not at all.

4. Why does Mamdani inference need an aggregation step?

Multiple rules can fire at the same time and may target the same output set at different strengths. Aggregation combines all clipped output sets into one before defuzzification so no rule's contribution is lost.

5. Why is defuzzification necessary?

The inference produces a fuzzy set, but a motor controller needs a single number. Defuzzification extracts one crisp value from the fuzzy set, typically the centroid.

6. What are the limitations of this system?

Membership functions are set by hand and are hard to tune. The rule base has gaps. The system cannot adapt from experience. Adding more inputs quickly increases the number of rules needed.

7. How could this system be validated in a real robotic application?

Test each rule individually with inputs that clearly match its conditions. Run the system in a simulator with sensor noise and measure collision rate and speed smoothness. Compare outputs to decisions made by a human operator under the same conditions.